Dropdown Menu Label: "How Can We Help?"

- I want to Redo My Driveway

- I want a Heated Driveway

- My driveway is Sinking or Cracked

- My backyard is a Steep Slope

- My pool deck is Slippery or Hot

- I have Drainage / Water Issues

- I need Privacy from Neighbors

- I want an Outdoor Kitchen / BBQ

- I want a Fire Pit / Fireplace

- My front steps are Crumbling

- I want a Modern / Resort Look

- I hate Weeds in my Patio

- I need an Accessible Entrance

- I want Lighting / Night Ambiance

- Other / Custom Vision

Solution 1: Driveway Transformation & Interlock Engineering

The Problem: "My current driveway is cracked asphalt, heaving concrete, or old pavers that have sunk. I want a permanent replacement that boosts curb appeal."

The Engineering Challenge

A driveway in the Greater Toronto Area faces two massive threats: Heavy Vehicle Loads and Freeze-Thaw Cycles.

- The Asphalt Failure: Asphalt is a flexible pavement, but it oxidizes and becomes brittle. Once water penetrates a small crack, it freezes, expands, and destroys the surface.

- The "Budget" Paver Failure: Many contractors quote low prices by only digging 6 inches deep. On Ontario’s clay soil, this is insufficient. The clay absorbs water, heaves in winter, and mixes with the gravel base, causing the "tire ruts" you see in older driveways.

The JR Solution: The "Road-Standard" Foundation

We do not build "driveways"; we build residential roads. Our process exceeds standard building code requirements to ensure longevity.

1. Deep-Impact Excavation
We remove the old asphalt and excavate 12 to 18 inches into the subgrade (depending on soil analysis). This removes the organic, compressible topsoil and unstable clay that causes sinking.

2. Geotextile Soil Separation
This is the step budget contractors skip. We install a heavy-duty, woven Geotextile Fabric at the bottom of the excavation.

- Why: This acts like a filter. It prevents the unstable clay soil below from rising up and contaminating your clean gravel base. It stabilizes the ground effectively, distributing the weight of your vehicles.

3. High-Performance Bedding (HPB)
Instead of standard limestone screenings (which trap water), we often utilize High-Performance Bedding (HPB). This is a washed, clear angular stone that achieves 98% compaction immediately. It allows water to drain vertically, preventing the formation of ice lenses under your driveway that cause heaving.

4. 80mm Vehicular-Rated Pavers
We strictly install pavers that are 80mm (3.15 inches) thick or greater for driveways.

- Why: Thinner 60mm pavers are for patios. A driveway paver must withstand the torque of power-steering (turning wheels while stopped) and the weight of SUVs. We source high-density stones from our CornerStone and LuxeStone divisions that are resistant to de-icing chemicals and oil stains.

5. Structural Edge Restraints
The edges of a driveway are the weakest point. We install a Concrete Reinforced Soldier Course or heavy-duty Permaledge restraints. This acts as a structural rib, locking the pavers together so they cannot spread outward over time.

The Result

- Immediate ROI: A custom stone driveway is one of the highest ROI exterior renovations (adding 10-15% to perceived property value).

- Zero-Maintenance: No resealing asphalt every year. No weeding (thanks to our Polymeric Sand).

- Permeability: We can engineer the system to meet city "Soft Landscaping" bylaws, allowing you to widen your driveway legally where asphalt would be prohibited.

Solution 2: The Automated Heated Driveway

The Problem: "I am tired of shoveling snow, and I’m worried about ice accidents."

The Engineering Challenge

In the GTA, winter brings a mix of heavy snow and freezing rain. Manual shoveling is labor-intensive, and hiring a plow service comes with risks—metal plow blades scratch expensive stone, and pile-ups damage landscaping. Furthermore, using rock salt (Sodium Chloride) to melt ice chemically attacks the concrete matrix of pavers, leading to pitting and deterioration.

The JR Solution: Hydronic & Electric Snow Melt Systems

We install fully automated snow-melting infrastructure directly beneath your driveway and walkways.

- The Technology: We utilize PEX tubing (Hydronic) or Resistive Cables (Electric) embedded in the bedding layer of the pavement.

- Smart Sensors: The system is governed by pavement-mounted sensors that detect both moisture and temperature. The system only activates when it is actually snowing or freezing, ensuring maximum energy efficiency. It does not run 24/7.

- Insulation Layer: To prevent heat loss into the ground, we install high-compression rigid insulation boards beneath the heating elements, forcing 90% of the thermal energy upward to melt the snow.

The Result

- Zero Accumulation: Snow melts as it hits the ground.

- Stone Protection: You never need to use salt, preserving the color and finish of your driveway for decades.

- Safety: Elimination of "black ice" slip hazards on your front steps.

Solution 3: Driveway Stabilization (Fixing the "Sink")

The Problem: "My driveway has tire ruts, sunken spots, or my asphalt is cracking."

The Engineering Challenge

Driveways sink for one reason: Base Failure.
Most "budget" contractors excavate only 6-8 inches and dump gravel on top of clay soil. Over time, the clay mixes with the gravel (subgrade contamination), destroying the structural integrity. When a heavy SUV parks on this weakened spot, the stones sink, and ruts form.

The JR Solution: The "High-Performance" Base

We treat your driveway like a municipal road, not a garden path.

- Deep Excavation: We excavate a minimum of 12 to 18 inches (depending on soil analysis) to remove organic, compressible soil.

- Geotextile Separation: We install a heavy-duty woven geotextile fabric at the bottom of the trench. This acts as a barrier, preventing the clay subsoil from rising up and contaminating your clean gravel base.

- Structural Aggregate: We use Granular 'A' or High-Performance Bedding (HPB)—a washed, clear limestone chip that achieves 95-98% compaction. This creates a rigid, non-settling platform that distributes vehicle weight evenly.

- Edge Restraints: We pour concrete-reinforced edging or install structural plastic restraints along the perimeter to prevent the pavers from spreading outward over time.

The Result

A driveway that remains perfectly flat, resists tire tracking, and is backed by our Workmanship Warranty against settlement.

Solution 4: Slope Management & Retaining Walls

The Problem: "My backyard is a steep hill and I have no usable space."

The Engineering Challenge

A slope is wasted real estate. However, simply stacking stones to hold back earth is dangerous. Soil creates Hydrostatic Pressure—when it rains, the water-logged soil behind a wall becomes incredibly heavy, pushing the wall forward until it topples.

The JR Solution: Reinforced Earth Systems

We reclaim your land by building engineered terraces using natural Armor Stone or Architectural Blocks.

- Geogrid Reinforcement: For walls over 24 inches, we install layers of Geogrid—a high-strength synthetic mesh. This mesh is sandwiched between the wall blocks and extends back into the soil slope (4-6 feet). The weight of the soil on the mesh anchors the wall, making it structurally impossible to tip over.

- The "Drainage Chimney": We never backfill a wall with dirt. We install a 12-inch vertical column of clear drainage stone directly behind the wall face, wrapped in filter fabric.

- Weeping Tiles: At the base of the wall, we install a perforated drain pipe that carries water away to a suitable outlet.

The Result

We turn a steep hill into a series of flat, functional "outdoor rooms" (upper dining terrace, lower lounging garden) that significantly increase your property’s usable square footage and resale value.

Solution 5: Pool Deck Safety & Thermodynamics

The Problem: "My current pool deck gets scorching hot in summer and is slippery when wet."

The Engineering Challenge

A pool deck must function as a "barefoot zone."

- Heat: Dark, dense stones (like black granite or slate) absorb infrared radiation. On a 30°C day, black stone can reach 60°C+, burning feet.

- Slip: Polished or honed surfaces become skating rinks when splashed with pool water.

The JR Solution: Thermal-Optimized Material Selection

We leverage our LuxeStone inventory to select materials specifically geologically suited for pool environments.

- Travertine & Limestone: These sedimentary stones are naturally porous and light in color. They reflect sunlight and utilize "evaporative cooling" to remain comfortable to the touch, even in direct July sun.

- Textured Sintered Stone: We supply manufactured slabs with specific "R-Ratings" (Slip Resistance). We aim for an R11 rating for pool decks, which provides "micro-grip" texture that feels smooth to the hand but provides traction for wet feet.

- Coping Profiles: We install Bullnose (Rounded) coping for safety (no sharp edges for swimmers) or Rebated Square Edge for a modern look, mechanically bonded to the pool beam for stability.

The Result

A resort-style pool deck that is safe for children running and comfortable for sunbathing all day long.

Solution 6: Advanced Drainage & Grading

The Problem: "I have standing water on my patio or worry about a wet basement."

The Engineering Challenge

Water is the enemy of structures. If water pools on your patio, it accelerates freeze-thaw damage (cracking stones). If it pools against your house, it causes basement leaks.

The JR Solution: Laser-Guided Water Management

We solve drainage issues before we lay a single stone.

- Positive Grading: Using laser levels, we establish a strict 1% to 2% slope on all hard surfaces, directing water away from your foundation.

- Channel Drains: In areas where slope is difficult (e.g., between a garage and a house), we install sleek, stainless steel Linear Channel Drains that capture surface runoff and pipe it underground to the storm sewer or a dry well.

- Permeable Paver Systems: For low-lying areas, we can install Permeable Pavers with wide, stone-chip joints. These sit on an "Open-Graded" base of clear stone (no sand). Rainwater passes through the driveway instantly, entering a storage gallery of rock beneath, where it slowly dissipates into the subsoil.

The Result

A dry basement, no ice patches in winter, and a patio that dries minutes after a summer storm ends.

Solution 7: Privacy & Feature Walls

The Problem: "I feel exposed in my own backyard. I want seclusion from neighbors without building a generic wood fence."

The Design Challenge

In the dense neighborhoods of the GTA, standard 6-foot wood fences often feel claustrophobic and offer zero sound dampening. Furthermore, municipal bylaws strictly limit fence heights, making it difficult to achieve true privacy for raised decks or patios.

The JR Solution: Architectural Masonry & Privacy Screens

We create "Sanctuaries," not cages. We use stone to build structure and definition.

- Masonry Sound Barriers: Unlike wood, a stone feature wall has mass. It physically dampens neighborhood noise (traffic, AC units). We build engineered masonry walls using architectural block or natural stone veneer that act as a visual and acoustic shield.

- Structural Pillars: We install stone-clad pillars with integrated lighting. These allow us to span high-end privacy screens (frosted glass, laser-cut aluminum, or horizontal cedar) that feel like part of the home’s architecture, not an afterthought.

- The "Green" Wall: We design planter-wall hybrids. By building a raised stone masonry planter (24-30 inches high) and planting mature cedars or grasses inside, we effectively raise the "privacy height" to 8 or 9 feet while complying with fence bylaws (since the height comes from 'nature').

The Result

A private courtyard feel that blocks sightlines while maintaining an open, airy atmosphere.

Solution 8: The Outdoor Kitchen & Chef’s Station

The Problem: "I want to cook and entertain outside, but I hate running back and forth to the indoor kitchen."

The Engineering Challenge

Building a kitchen outdoors is not the same as building one indoors. Materials face -30°C winters, blazing UV sun, and rain.

- The Material Failure: Standard indoor quartz turns yellow in the sun. Wood framing rots against damp ground. Plywood delaminates.

The JR Solution: The "Zero-Decay" Structure

We build outdoor kitchens that are virtually indestructible.

- Masonry Structure: We do not use wood studs. We build the "carcass" of the island using concrete block or galvanized steel framing. It is rot-proof, fire-proof, and pest-proof.

- Utility Infrastructure: During the rough-in phase, we trench and run licensed Gas, Water, and Electrical lines to the island location. We include "Winterization Points" (shut-off valves and blow-out ports) so you can safely close the kitchen for winter without plumbing damage.

- Surface Technology: We strictly recommend Granite or Sintered Stone (from our CornerStone division) for countertops. These materials are thermally shock-resistant—you can place a hot pot directly on them or let snow pile up without risk of cracking.

The Result

A fully functional culinary zone with built-in Napoleon/Weber grills, refrigeration, and bar seating that serves as the social heart of your summer.

Solution 9: Fire Pits & Fireplaces

The Problem: "My backyard is unusable once the sun goes down or the weather turns chilly."

The Design Challenge

Fire brings people together, but safety is paramount. A cheap metal bowl kills the grass and rusts. A poorly built stone pit can crack from thermal stress (heat expanding the stone while the outside air is cold).

The JR Solution: Refractory Engineering

We build fire features that are safe, smokeless (if gas), and permanent.

- The Core: We construct the inner shell using Fire Brick and Refractory Mortar (the same material used in kilns). This absorbs the intense heat, protecting the exterior decorative stone from cracking.

- Gas Integration: For instant ambiance, we install 300,000+ BTU gas burners (Natural Gas or Propane). We conceal the plumbing beneath the patio base, emerging invisibly inside the fire structure.

- Safety Clearances: We design the feature to meet all local fire codes regarding distance from combustibles (fences, siding). For wood-burning features, we can integrate spark arrestors and wind guards.

The Result

A 3-season outdoor living room. You gain 2-3 extra months of backyard use in Spring and Autumn.

Solution 10: Front Step Restoration (Crumbling Concrete)

The Problem: "My front concrete steps are pitting, spalling, or separating from the house. It looks neglected and unsafe."

The Engineering Challenge

Concrete steps fail because of Salt Damage and Frost Heave. Once water gets into a hairline crack and freezes, it acts like a wedge, popping the face of the concrete off (spalling).

The JR Solution: Natural Stone Overlay or Reconstruction

We have two methods to restore grandeur to your entrance.

- Method A: The Stone Overlay (Veneering): If the structural concrete core is sound, we don't need to demolish it. We treat the concrete with a bonding agent and clad it in 2-inch thick Natural Stone Treads (Limestone or Sandstone) and vertical veneers. This gives the look of solid stone construction at a lower cost than a rebuild.

- Method B: The Solid Stone Rebuild: For ultimate longevity, we remove the concrete entirely. We stack massive, solid blocks of stone (4-6 feet wide).

  - The Advantage: A solid block has no mortar joints to crack and no veneer to pop off. It relies on gravity and friction. It is effectively immune to winter damage.

The Result

A grand entrance that enhances curb appeal and provides safe, non-slip footing for delivery drivers and guests.

Solution 11: The "Modern Resort" Look

The Problem: "I hate the look of small, busy 'cobblestone' pavers. I want the seamless, clean look of a Miami hotel or European villa."

The Design Challenge

Modern design relies on Scale and Minimalism. Achieving this requires large format slabs (e.g., 24x36 or 36x36 inches). However, large slabs are unforgiving. If the base isn't perfect, they will rock, crack, or have "lippage" (uneven edges that you trip on).

The JR Solution: Large Format Sintered Stone & Precision Bedding

We utilize advanced installation techniques required for modern materials.

- Sintered Stone / Porcelain: We use high-tech manufactured slabs that mimic the look of marble or concrete but are non-porous and stain-proof.

- Open-Graded Base: We install these on a chip-stone base (HPB) that allows for perfect leveling.

- Lippage Tuning: We use spacer-leveling clips during installation. These clips mechanically lock adjacent slabs at the exact same height while the bedding settles, ensuring a perfectly flat, glass-like surface plane.

- Tight Tolerances: We aim for 3mm joints (barely visible), filled with resin-based grout for a seamless, monolithic appearance.

The Result

A sleek, contemporary outdoor floor that blurs the line between indoor luxury and outdoor living.

Solution 12: The Weed-Free Patio System

The Problem: "I love the look of stone, but I hate spending my weekends pulling weeds and fighting ant hills in the joints."

The Engineering Truth

A common myth is that weeds grow from under the patio. In reality, 95% of weeds grow from the top down. Seeds blow in the wind, land in the cracks, and take root in the dirt that accumulates there.
Standard "playground sand" washes away in the rain, leaving deep gaps that become perfect incubators for moss, weeds, and ant colonies.

The JR Solution: Polymer Joint Stabilization

We treat the joints as a structural element, not an afterthought. We utilize High-Performance Polymeric Sand (such as HP NextGel or G2).

- The Chemistry: This material creates a cross-linked polymer bond when activated with water. It cures into a unified, flexible substance that feels like hard rubber.

- The "Elastic" Bond: Unlike mortar (which cracks when the ground moves), polymeric sand offers flexibility. It absorbs the micro-movements of the Canadian freeze-thaw cycle without breaking its seal.

- Rapid Rain Technology: The specific polymers we use become rain-safe within minutes of installation, preventing the "washout" that ruins cheaper jobs.

The Result

A sealed, impermeable surface. Weeds cannot root, ants cannot burrow, and the clean lines of your patio remain crisp for years.

Solution 13: Barrier-Free & Accessible Entrances

The Problem: "I need wheelchair, walker, or stroller access, but I don't want my home to look like a hospital or have an ugly metal ramp."

The Design Challenge

Accessibility usually comes at the cost of aesthetics. Standard aluminum ramps are functional but destroy curb appeal. Furthermore, standard code requires a 1:12 slope ratio (1 inch of rise for every 12 inches of length), which requires significant landscape engineering to hide.

The JR Solution: Universal "Invisible" Design

We believe dignity and design go hand-in-hand. We integrate accessibility features so seamlessly that guests don't realize they are "accessible"—they just feel welcoming.

- Integrated Stone Ramps: Instead of metal, we build ramps using the same premium pavers and retaining walls as the rest of the landscape. We utilize curved masonry walls to mask the grade change, turning the ramp into a winding garden path.

- Texture Mapping: We select stones with specific tactile finishes (from our LuxeStone inventory) that provide maximum grip for wheels and walkers, even in rain.

- Seamless Thresholds: We engineer the grading to eliminate the "lip" at the front door, waterproofing the transition to allow for a zero-step entry that keeps water out but lets wheels roll in smoothly.

The Result

A "Forever Home" exterior that creates safe, independent access for aging parents or family members with mobility challenges, without compromising on luxury.

Solution 14: Integrated Landscape Lighting

The Problem: "My house looks great during the day, but at night, the steps are dangerous and the backyard looks like a black hole."

The Engineering Challenge

Lighting is often an afterthought. If you try to add lights after the stone is laid, you have unsightly wires running across the patio.
Solar lights are not the answer—they lack the lumen output (brightness) and battery life to provide safety during Canadian winters.

The JR Solution: Hardwired Low-Voltage Infrastructure

We design lighting plans during the Excavation Phase.

- The "Rough-In": We bury conduit piping and wiring beneath the gravel base of your driveway and patio before compaction. This keeps the system protected and invisible.

- Core-Drilling: We drill directly into the stone blocks.

  - Step Lights: Recessed into the risers for safety.

  - Cap Lights: Hidden under the lip of retaining walls to wash the stone texture with light.

  - In-Grade Lights: Flush-mounted into the driveway to mark the edges for parking.

- Smart Control: We install professional-grade transformers that link to your smartphone. You can set schedules (e.g., "On at Sunset, Off at 11 PM") or dim zones separately for entertaining.

The Result

Instant Ambiance & Security. Your property looks larger at night because the light pushes back the darkness. More importantly, you eliminate trip hazards, making your outdoor space usable 24 hours a day.

Solution 15: Bespoke Design & Complex Builds

The Problem: "I have a specific vision (or a set of architect’s drawings) that doesn't fit into a standard box. I need a partner who can execute a complex, multi-layered project."

The Construction Challenge

Standard contractors operate on templates. When a project involves unique constraints—such as heritage property restrictions, custom stone fabrication, intricate water features, or multi-level engineering—template contractors cut corners or refuse the job.
True custom work requires Cross-Disciplinary Coordination: the ability to merge engineering, artistry, and logistics seamlessly.

The JR Solution: The "Blank Canvas" Approach

We thrive on complexity. Because we are part of the JR Partners ecosystem, we have resources that standalone landscapers do not.

- Custom Fabrication: If the stone you want doesn't exist in a catalog, our CornerStone division can fabricate it. We can cut custom radius curves, engrave family crests, or create monolithic stone features to your exact specifications.

- Architectural Collaboration: We speak the language of architects. If you are working with a Landscape Architect or Interior Designer, we act as the Technical Executor. We review their drawings for structural feasibility and bring their vision to life with millimeter precision.

- Total Site Management: For large estates or commercial plazas, we manage the entire "envelope." This includes coordination of gas, electrical, irrigation, and softscaping (planting) alongside the masonry work. You deal with one Project Manager, not five different trades.

Contact Us (link to contact page)